# Fitness Band Telemetry: Data Format Specification

FitFuel — CMPE 195A/B Senior Design | San José State University | Spring 2026

Ishan Sikka

# Purpose

This document specifies

in what format telemetry data are represented when transmitted from the fitness band to a host system (mobile device or personal computer). It complements the parallel summary on sensor versus summary data authored by Tanish: here the focus is exclusively on encoding, layout, and host-side interpretation—not on whether samples are raw or aggregated at the source.

# Transport and encapsulation

Telemetry is delivered over Bluetooth Low Energy using GATT notifications on a project-defined Telemetry characteristic within a custom service. The application payload is a binary record, not JSON or XML; compact fixed-width fields minimize airtime and parsing cost. Service and characteristic 128-bit UUIDs will be published in a shared constants module referenced by firmware and client software (see project connectivity documentation).

# Binary layout (application payload)

All multi-byte integers use little-endian byte order. The notification payload is structured as follows (approximately twelve octets).

| Byte offset | Field | Type | Semantics |
| --- | --- | --- | --- |
| 0 | Packet type | uint8 | Message class (e.g., 0x01 = telemetry) |
| 1–2 | Heart rate | uint16 | Beats per minute |
| 3–4 | Steps | uint16 | Step count since reset |
| 5–8 | Calories | uint32 | Energy as kilocalories × 10 (divide by 10 for kcal) |
| 9 | Battery | uint8 | State of charge, 0–100% |
| 10–11 | Sequence | uint16 | Monotonic counter; gaps imply missed notifications |

This record fits typical default ATT MTU limits; larger payloads are possible after MTU negotiation between Central and Peripheral.

# Host software interpretation

On Android, the GATT callback supplies the characteristic value as a byte array; fields are read using unsigned little-endian interpretation consistent with the table above. On desktop systems, an equivalent layout may be parsed with Python struct unpacking—for example, format string <B H H I B H in little-endian order (12 bytes total)—when using a BLE library such as bleak.

Standard Bluetooth Battery Service characteristics, if exposed, follow Bluetooth SIG data types and are orthogonal to this custom telemetry record.

# Reference

docs/hardware-fitness-band-connectivity.md